| dgemm_def | dgemm_transpose |  | dgemm_block |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time, s | Time, s | Speedup | bs = 8 |  | bs = 16 |  | bs = 32 |  | bs = 4 |  |
| 1.68 | 0.37 | 4.54 | Time, s | Speedup | Time, s | Speedup | Time, s | Speedup | Time, s | Speedup |
|  |  |  | 0.79 | 2.12 | 0.71 | 2.36 | 0.75 | 2.24 | 1.12 | 1.5 |

время для dgemm unroll 0.243519

для dgemm_def

--1530475-- warning: L3 cache found, using its data for the LL simulation.

# DGEMM def: N=1024, elapsed time (sec) 29.400996

==1530475==

==1530475== I   refs:      13,054,171,539

==1530475== I1  misses:             1,493

==1530475== LLi misses:             1,486

==1530475== I1  miss rate:           0.00%

==1530475== LLi miss rate:           0.00%

==1530475==

==1530475== D   refs:       3,280,003,598  (3,260,589,118 rd   + 19,414,480 wr)

==1530475== D1  misses:     1,617,281,352  (1,615,314,605 rd   +  1,966,747 wr)

==1530475== LLd misses:           922,675  (      528,827 rd   +    393,848 wr)

==1530475== D1  miss rate:           49.3% (         49.5%     +       10.1%  )

==1530475== LLd miss rate:            0.0% (          0.0%     +        2.0%  )

==1530475==

==1530475== LL refs:        1,617,282,845  (1,615,316,098 rd   +  1,966,747 wr)

==1530475== LL misses:            924,161  (      530,313 rd   +    393,848 wr)

==1530475== LL miss rate:             0.0% (          0.0%     +        2.0%  )

perf

# DGEMM def: N=1024, elapsed time (sec) 1.702197

[ perf record: Woken up 2 times to write data ]

[ perf record: Captured and wrote 0.166 MB perf.data (4317 samples) ]

для dgemm_interchange

--1530987-- warning: L3 cache found, using its data for the LL simulation.

# DGEMM interchange: N=1024, elapsed time (sec) 14.099675

==1530987==

==1530987== I   refs:      7,026,204,464

==1530987== I1  misses:            1,493

==1530987== LLi misses:            1,486

==1530987== I1  miss rate:          0.00%

==1530987== LLi miss rate:          0.00%

==1530987==

==1530987== D   refs:      3,283,149,484  (2,458,428,587 rd   + 824,720,897 wr)

==1530987== D1  misses:      403,835,208  (  403,441,324 rd   +     393,884 wr)

==1530987== LLd misses:        1,051,756  (      657,907 rd   +     393,849 wr)

==1530987== D1  miss rate:          12.3% (         16.4%     +         0.0%  )

==1530987== LLd miss rate:           0.0% (          0.0%     +         0.0%  )

==1530987==

==1530987== LL refs:         403,836,701  (  403,442,817 rd   +     393,884 wr)

==1530987== LL misses:         1,053,242  (      659,393 rd   +     393,849 wr)

==1530987== LL miss rate:            0.0% (          0.0%     +         0.0%  )

perf

# DGEMM interchange: N=1024, elapsed time (sec) 0.380053

[ perf record: Woken up 2 times to write data ]

[ perf record: Captured and wrote 0.130 MB perf.data (3376 samples) ]

для dgemm_block

--1531059-- warning: L3 cache found, using its data for the LL simulation.

# DGEMM bloc: N=1024, BS=32, elapsed time (sec) 49.771503

==1531059==

==1531059== I   refs:      33,406,252,937

==1531059== I1  misses:             1,503

==1531059== LLi misses:             1,496

==1531059== I1  miss rate:           0.00%

==1531059== LLi miss rate:           0.00%

==1531059==

==1531059== D   refs:       9,820,267,700  (6,581,200,435 rd   + 3,239,067,265 wr)

==1531059== D1  misses:       428,238,409  (  427,844,528 rd   +       393,881 wr)

==1531059== LLd misses:           797,845  (      403,999 rd   +       393,846 wr)

==1531059== D1  miss rate:            4.4% (          6.5%     +           0.0%  )

==1531059== LLd miss rate:            0.0% (          0.0%     +           0.0%  )

==1531059==

==1531059== LL refs:          428,239,912  (  427,846,031 rd   +       393,881 wr)

==1531059== LL misses:            799,341  (      405,495 rd   +       393,846 wr)

==1531059== LL miss rate:             0.0% (          0.0%     +           0.0%  )

perf

# DGEMM bloc: N=1024, BS=32, elapsed time (sec) 0.755897

[ perf record: Woken up 2 times to write data ]

[ perf record: Captured and wrote 0.045 MB perf.data (1146 samples) ]

2 задание

время об версии 1.095

время опт 0.35

iv321s11@pine:~/pvt/pct-spring-lab1/branch-image-dark$ ./a.out

Time 0.353189, dark 0

iv321s11@pine:~/pvt/pct-spring-lab1/branch-image-dark$ gcc prog.c

iv321s11@pine:~/pvt/pct-spring-lab1/branch-image-dark$ ./a.out

Time 1.097748, dark 0

iv321s11@pine:~/pvt/pct-spring-lab1/branch-image-dark$ gcc -O2 prog.c

iv321s11@pine:~/pvt/pct-spring-lab1/branch-image-dark$ ./a.out

Time 0.060416, dark 0

iv321s11@pine:~/pvt/pct-spring-lab1/branch-image-dark$

3 задание

iv321s11@pine:~/pvt/pct-spring-lab1/pipeline-loop-unrolling$ gcc prog.c

iv321s11@pine:~/pvt/pct-spring-lab1/pipeline-loop-unrolling$ ./a.out

Time 0.266723, sum 100000000

Time 0.158925, sum 100000000

Time 0.106798, sum 100000000

iv321s11@pine:~/pvt/pct-spring-lab1/pipeline-loop-unrolling$ gcc -O2 prog.c

iv321s11@pine:~/pvt/pct-spring-lab1/pipeline-loop-unrolling$ ./a.out

Time 0.058609, sum 100000000

Time 0.052970, sum 100000000

Time 0.049114, sum 100000000